Software Report

The numerous facets of the software employed in our project are described in this software report. Our project’s software is designed as best as possible to assure a smooth running of our smartmeter device. The microcontroller uses an analogue-to-digital converter to measure the busbar voltage and current, wind capacity, and solar capacity (ADC). Based on the matching load calls, the microcontroller (il Matto) examines the measurements to decide which load switch should be turned on and off. The algorithm also assesses how much current is required from main if renewable energy is inadequate to fully charge the battery and power the load.

The software design is written in C, which was chosen for the design’s adaptability, portability, and embedded system effectiveness. Analogue to Digital conversion (ADC), Pulse Width Modulation (PWM), display, and algorithm are the system’s four core parts.

The busbar’s voltage and current, as well as the amounts of wind and solar energy, are all measured by the ADC. To fit the ADC’s range, the signals are scaled down using and external circuit interface. To calculate and control power, the module converts the analogue signals to digital signals. Traditionally, the busbar voltage and current signals are stabilised using complex circuit interfaces before they can be fed into a microcontroller for further reading. However, in our smartmeter, we have implemented a unique approach where the il Matto is used to directly measure the AC signals without converting it to DC. Many advantages of this approach include a more straightforward overall design and a circuit interface with less complexity.

The quantity of power requested from the main supply is decided by the PWM signal. It adjusts the voltage levels to the proper level required to charge the battery and power the load.

The display module is in responsible for providing the user of the real-time system’s status. This includes power consumption, battery status and percentage, and load status. A simple graphic depiction of the power produced by renewable sources is also included.

Based on the appropriate load calls, the algorithm module determines which load switches to turn on and off. If there is insufficient energy from renewable sources to charge the battery and power the load, it will decide how much power to request from main supply or if there is enough battery percentage to discharge. The algorithm is also designed so that to increase the survival rate as much as possible by deciding which is the best load to turn off in the event of insufficient renewable energy, main supply, and battery to power all the load. All possible outcomes is considered and the best way to disable the load is chosen.