TEAM B – SMART-METER DEVICE

UNIVERSITY OF SOUTHAMPTON

Description

A low cost, smart electricity meter that is capable of maximising your resources. With low power consumption, this smart meter can measure the incoming renewable energy and mains and make decisions based on this to produce a reliable energy output suitable to manage a microgrid.

User Guide

- Powering

  - The device powers automatically

  - Plug the cable into a socket

- Connections

  - 4 ethernet connectors

  - Port 1: Analogue Input - Mains capacity

  - Port 2: Analogue Output - Renewable energy

  - Port 3: Digital Input - Load and battery control

  - Port 4: Digital Output - Load demand

- Disassembly

  - Unscrew the two screws on the side of the top part, Screw A and Screw B

  - Split the top part and the bottom part

- Display

- LED

  - A low current green LED - used to indicate the operation of the device

Power Supply Unit

- Components List

| Component Name | Value/Serial Number | Unit |
| --- | --- | --- |
| X2 rated Capacitor | 1.5uF/x | 1 |
| Capacitor | 6800uF/ | 1 |
| Resistor 3 | 680k Ohms/x | 1 |
| Resistor 4 | 1k Ohms/x | 1 |
| Resistor 1 | 82 Ohms/x | 1 |
| Resistor 2 | 53 Ohms /x | 1 |
| Diodes | x/1N14148 | 4 |
| Zener Diode | 12V/TFZV12B | 1 |

- Specification

| Model | TEAM-B-SamrtMeter3000 |
| --- | --- |
| Input Voltage | 230V@50Hz (UK standard) |
| Input current | 75mA |
| Power consumption | 1.5W/0.0015KWh |
| Output power | 0.9W |
| Efficiency | 60% |

.

Control

- Microcontroller

  - IlMatto Board with ATMega644PA IC.

  - Algorithm-based control

Structure

- Materials

  - Walls and interiors: Plastic

  - Gaskets in between parts: Rubber

  - Mounting lug and display cover: Acrylic

  - M3 and M2.5 screws

- Features

  - Solid and resistant

  - Water resistant to IP55 standards

  - Safe against shock hazards

- Dimensions

  - Box 105x105x105 mm

  - 4 ports 14.50x15.25 mm

  - Display 74.20x53.40 mm